LAB REPORT

RIP and EIGRP Redistribution with VLANs, DHCP, DNS and Web Server

OBJECTIVES

- Configure two routers, R1_RIP and R2_EIGRP, using RIP Version 2 and EIGRP AS 100 in different routing domains.

- Implement two-way route redistribution between RIP and EIGRP for full communication between both networks.

- Configure VLANs 10, 20 and 30 on Layer 2 switches for traffic separation.

- Configure router-on-a-stick using 802.1Q trunking so that multiple VLANs can communicate through one router interface.

- Deploy DHCP servers and use ip helper-address for dynamic IP assignment across VLANs.

- Configure DNS and HTTP services and verify end-to-end connectivity using domain name and web access.

- Observe routing behaviour and verify successful redistribution of routes between the two protocols.

COMPONENTS REQUIRED

| Device | Qty | Hostname | Role |
| --- | --- | --- | --- |
| Router-PT (Gigabit) | 2 | R1_RIP, R2_EIGRP | RIP and EIGRP routers |
| Cisco 2960-24TT | 2 | SW1_RIP, SW2_EIGRP | Layer 2 VLAN switches |
| Server-PT | 4 | DHCP1, DHCP2, DNS, Web | DHCP, DNS and HTTP services |
| PC-PT | 4 | PC1_VLAN10, PC2_VLAN20, PC3_VLAN10, PC4_VLAN20 | End-user hosts |

Protocols Used

- RIP Version 2

- EIGRP AS 100

- Route Redistribution

- IEEE 802.1Q VLAN Trunking

- DHCP with IP Helper-Address

- DNS and HTTP

THEORY

RIP Version 2 is a classless distance-vector routing protocol that uses hop count as its metric. It supports VLSM and CIDR and is suitable for small networks. EIGRP is an advanced Cisco routing protocol that uses the DUAL algorithm and provides fast convergence. It uses bandwidth and delay in its composite metric and supports efficient route updates.

In this lab, RIP is used on the left network (R1_RIP side) and EIGRP is used on the right network (R2_EIGRP side). Since both routing protocols are different, route redistribution is required so that networks learned by RIP can be shared with EIGRP and networks learned by EIGRP can be shared with RIP.

VLANs are used to divide a switch into multiple logical networks. Because several VLANs exist on each side but each router has only one LAN physical interface, router-on-a-stick is used. In this method, one physical router interface is divided into multiple subinterfaces, and each subinterface acts as the default gateway for one VLAN using 802.1Q encapsulation.

DHCP is used to assign IP addresses dynamically to hosts. Since the DHCP server is located in a different VLAN from some clients, ip helper-address is used on router subinterfaces to forward DHCP broadcast requests to the correct DHCP server. DNS resolves domain names to IP addresses, and the web server tests application-level connectivity.

NETWORK DESIGN

VLAN Plan

| Side | VLAN | Network | Purpose |
| --- | --- | --- | --- |
| RIP Side | VLAN 10 | 192.168.10.0/24 | Users |
| RIP Side | VLAN 20 | 192.168.20.0/24 | Users |
| RIP Side | VLAN 30 | 192.168.30.0/24 | Servers |
| EIGRP Side | VLAN 10 | 172.16.10.0/24 | Users |
| EIGRP Side | VLAN 20 | 172.16.20.0/24 | Users |
| EIGRP Side | VLAN 30 | 172.16.30.0/24 | Servers |
| WAN Link | --- | 10.10.10.0/30 | Router to Router |

IP ADDRESSING SCHEME

| Device | Interface | IP Address | Subnet Mask | Gateway |
| --- | --- | --- | --- | --- |
| R1_RIP | Gig0/0/0.10 | 192.168.10.1 | 255.255.255.0 | --- |
| R1_RIP | Gig0/0/0.20 | 192.168.20.1 | 255.255.255.0 | --- |
| R1_RIP | Gig0/0/0.30 | 192.168.30.1 | 255.255.255.0 | --- |
| R1_RIP | Gig0/0/1 | 10.10.10.1 | 255.255.255.252 | --- |
| R2_EIGRP | Gig0/0/0.10 | 172.16.10.1 | 255.255.255.0 | --- |
| R2_EIGRP | Gig0/0/0.20 | 172.16.20.1 | 255.255.255.0 | --- |
| R2_EIGRP | Gig0/0/0.30 | 172.16.30.1 | 255.255.255.0 | --- |
| R2_EIGRP | Gig0/0/1 | 10.10.10.2 | 255.255.255.252 | --- |
| DHCP1_Server | Fa0 | 192.168.30.10 | 255.255.255.0 | 192.168.30.1 |
| DHCP2_Server | Fa0 | 172.16.30.10 | 255.255.255.0 | 172.16.30.1 |
| DNS_Server | Fa0 | 172.16.30.20 | 255.255.255.0 | 172.16.30.1 |
| Web_Server | Fa0 | 172.16.30.30 | 255.255.255.0 | 172.16.30.1 |
| PC1_VLAN10 | Fa0 | DHCP | --- | 192.168.10.1 |
| PC2_VLAN20 | Fa0 | DHCP | --- | 192.168.20.1 |
| PC3_VLAN10 | Fa0 | DHCP | --- | 172.16.10.1 |
| PC4_VLAN20 | Fa0 | DHCP | --- | 172.16.20.1 |

PROCEDURE

Step 1: Configure SW1_RIP

enable

configure terminal

hostname SW1_RIP

vlan 10

name VLAN10_USERS

exit

vlan 20

name VLAN20_USERS

exit

vlan 30

name VLAN30_SERVERS

exit

interface fastEthernet 0/1

description Trunk_to_R1

switchport mode trunk

exit

interface fastEthernet 0/2

description DHCP1_Server

switchport mode access

switchport access vlan 30

exit

interface fastEthernet 0/3

description PC1_VLAN10

switchport mode access

switchport access vlan 10

exit

interface fastEthernet 0/4

description PC2_VLAN20

switchport mode access

switchport access vlan 20

exit

interface vlan 10

ip address 192.168.10.2 255.255.255.0

no shutdown

exit

ip default-gateway 192.168.10.1

end

write memory

Step 2: Configure SW2_EIGRP

enable

configure terminal

hostname SW2_EIGRP

vlan 10

name VLAN10_USERS

exit

vlan 20

name VLAN20_USERS

exit

vlan 30

name VLAN30_SERVERS

exit

interface fastEthernet 0/1

description Trunk_to_R2

switchport mode trunk

exit

interface fastEthernet 0/2

description DHCP2_Server

switchport mode access

switchport access vlan 30

exit

interface fastEthernet 0/3

description DNS_Server

switchport mode access

switchport access vlan 30

exit

interface fastEthernet 0/4

description Web_Server

switchport mode access

switchport access vlan 30

exit

interface fastEthernet 0/5

description PC3_VLAN10

switchport mode access

switchport access vlan 10

exit

interface fastEthernet 0/6

description PC4_VLAN20

switchport mode access

switchport access vlan 20

exit

interface vlan 10

ip address 172.16.10.2 255.255.255.0

no shutdown

exit

ip default-gateway 172.16.10.1

end

write memory

Step 3: Configure R1_RIP

enable

configure terminal

hostname R1_RIP

interface GigabitEthernet0/0/0

no ip address

no shutdown

exit

interface GigabitEthernet0/0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

ip helper-address 192.168.30.10

exit

interface GigabitEthernet0/0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

ip helper-address 192.168.30.10

exit

interface GigabitEthernet0/0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

exit

interface GigabitEthernet0/0/1

ip address 10.10.10.1 255.255.255.252

no shutdown

exit

router rip

version 2

network 192.168.10.0

network 192.168.20.0

network 192.168.30.0

network 10.10.10.0

no auto-summary

redistribute eigrp 100 metric 5

exit

router eigrp 100

network 10.10.10.0 0.0.0.3

no auto-summary

redistribute rip metric 10000 100 255 1 1500

exit

end

write memory

Step 4: Configure R2_EIGRP

Note: Router uses GigabitEthernet0/0/0 (LAN) and GigabitEthernet0/0/1 (WAN).

enable

configure terminal

hostname R2_EIGRP

interface GigabitEthernet0/0/0

no ip address

no shutdown

exit

interface GigabitEthernet0/0/0.10

encapsulation dot1Q 10

ip address 172.16.10.1 255.255.255.0

ip helper-address 172.16.30.10

exit

interface GigabitEthernet0/0/0.20

encapsulation dot1Q 20

ip address 172.16.20.1 255.255.255.0

ip helper-address 172.16.30.10

exit

interface GigabitEthernet0/0/0.30

encapsulation dot1Q 30

ip address 172.16.30.1 255.255.255.0

exit

interface GigabitEthernet0/0/1

ip address 10.10.10.2 255.255.255.252

no shutdown

exit

router eigrp 100

network 172.16.10.0 0.0.0.255

network 172.16.20.0 0.0.0.255

network 172.16.30.0 0.0.0.255

network 10.10.10.0 0.0.0.3

no auto-summary

redistribute rip metric 10000 100 255 1 1500

exit

router rip

version 2

network 10.10.10.0

no auto-summary

redistribute eigrp 100 metric 5

exit

end

write memory

Step 5: Configure DHCP1_Server

Desktop > IP Configuration

| Field | Value |
| --- | --- |
| IP Address | 192.168.30.10 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 192.168.30.1 |
| DNS Server | 172.16.30.20 |

Services > DHCP > ON — Delete default pool, then create:

| Pool Name | Default Gateway | Start IP |
| --- | --- | --- |
| VLAN10 | 192.168.10.1 | 192.168.10.50 |
| VLAN20 | 192.168.20.1 | 192.168.20.50 |

Step 6: Configure DHCP2_Server

Desktop > IP Configuration

| Field | Value |
| --- | --- |
| IP Address | 172.16.30.10 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 172.16.30.1 |
| DNS Server | 172.16.30.20 |

Services > DHCP > ON — Delete default pool, then create:

| Pool Name | Default Gateway | Start IP |
| --- | --- | --- |
| VLAN10 | 172.16.10.1 | 172.16.10.50 |
| VLAN20 | 172.16.20.1 | 172.16.20.50 |

Step 7: Configure DNS_Server

Desktop > IP Configuration

| Field | Value |
| --- | --- |
| IP Address | 172.16.30.20 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 172.16.30.1 |
| DNS Server | 172.16.30.20 |

Services > DNS > ON — Add records:

| Name | Address |
| --- | --- |
| www.networklab.local | 172.16.30.30 |
| networklab.local | 172.16.30.30 |

Step 8: Configure Web_Server

Desktop > IP Configuration

| Field | Value |
| --- | --- |
| IP Address | 172.16.30.30 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 172.16.30.1 |
| DNS Server | 172.16.30.20 |

Services > HTTP > ON — Edit index.html:

<html>

<head><title>Network Lab</title></head>

<body>

<h1>Welcome to RIP and EIGRP Redistribution Lab done by utsav bhatta.</h1>

<p>DNS name: www.networklab.local</p>

</body>

</html>

Step 9: Configure PCs

Click each PC > Desktop > IP Configuration > select DHCP

| PC | Expected IP After DHCP |
| --- | --- |
| PC1_VLAN10 | 192.168.10.50 |
| PC2_VLAN20 | 192.168.20.50 |
| PC3_VLAN10 | 172.16.10.50 |
| PC4_VLAN20 | 172.16.20.50 |

VERIFICATION

On R1_RIP — type end first, then:

show ip route

show ip protocols

show ip eigrp neighbors

On R2_EIGRP — type end first, then:

show ip route

show ip protocols

show ip eigrp neighbors

VERIFICATION ON SWITCHES:

On Switches

show vlan brief

show interfaces trunk

SW_RIP:

SW_EIGRP:

On PCs — Desktop > Command Prompt

ipconfig

ping 192.168.10.1

ping 172.16.30.30

PING TEST IN PC1_VLAN10:

Browser test in PC3_VLAN10: Desktop > Web Browser

OBSERVATION

Routing Table on R1_RIP

| Route Code | Network | Learned From |
| --- | --- | --- |
| C | 192.168.10.0/24 | Directly connected |
| C | 192.168.20.0/24 | Directly connected |
| C | 192.168.30.0/24 | Directly connected |
| C | 10.10.10.0/30 | Directly connected |
| D EX | 172.16.10.0/24 | Redistributed from EIGRP |
| D EX | 172.16.20.0/24 | Redistributed from EIGRP |
| D EX | 172.16.30.0/24 | Redistributed from EIGRP |

Routing Table on R2_EIGRP

| Route Code | Network | Learned From |
| --- | --- | --- |
| C | 172.16.10.0/24 | Directly connected |
| C | 172.16.20.0/24 | Directly connected |
| C | 172.16.30.0/24 | Directly connected |
| C | 10.10.10.0/30 | Directly connected |
| R | 192.168.10.0/24 | Redistributed from RIP |
| R | 192.168.20.0/24 | Redistributed from RIP |
| R | 192.168.30.0/24 | Redistributed from RIP |

VLAN Assignment on Switches

| Switch | Port | VLAN | Device |
| --- | --- | --- | --- |
| SW1_RIP | Fa0/1 | Trunk | R1_RIP |
| SW1_RIP | Fa0/2 | 30 | DHCP1_Server |
| SW1_RIP | Fa0/3 | 10 | PC1_VLAN10 |
| SW1_RIP | Fa0/4 | 20 | PC2_VLAN20 |
| SW2_EIGRP | Fa0/1 | Trunk | R2_EIGRP |
| SW2_EIGRP | Fa0/2 | 30 | DHCP2_Server |
| SW2_EIGRP | Fa0/3 | 30 | DNS_Server |
| SW2_EIGRP | Fa0/4 | 30 | Web_Server |
| SW2_EIGRP | Fa0/5 | 10 | PC3_VLAN10 |
| SW2_EIGRP | Fa0/6 | 20 | PC4_VLAN20 |

RESULT
The project was completed successfully. RIP and EIGRP networks were connected through two-way redistribution. VLAN communication, DHCP, DNS, and web services worked properly, and devices from both sides communicated successfully.

DISCUSSION
This project showed how different routing protocols can work together using route redistribution. It also demonstrated the use of router-on-a-stick for VLANs and DHCP relay for automatic IP assignment across networks. Proper metrics and correct configuration were necessary for successful communication.

CONCLUSION
The project proved that RIP and EIGRP can be integrated into one working network. With proper VLAN setup, redistribution, and network services, full end-to-end connectivity was achieved.